Commerce Control List	Supplement No. 1 to Part 774	Category 4—page 1

CATEGORY 4 - COMPUTERS

Note 1: Computers, related equipment and “software” performing telecommunications or “local area network” functions must also be

evaluated against the performance characteristics of Category 5, Part 1 (Telecommunications).

Note 2 : Control units that directly interconnect the buses or channels of central processing units, ‘main storage’ or disk

controllers are not regarded as telecommunications equipment described in Category 5, Part 1 (Telecommunications).

N.B.: For the control status of “software” “specially designed” for packet switching, see ECCN 5D001. (Telecommunications).

Technical Note: ‘Main storage’ is the primary storage for data or instructions for rapid access by a central processing unit. It consists of the internal storage of a “digital computer” and any hierarchical extension thereto, such as cache storage or non-sequentially accessed extended storage.

- “END ITEMS,” “EQUIPMENT,” “ACCESSORIES,” “ATTACHMENTS,”

“PARTS,” “COMPONENTS,” AND

“SYSTEMS”

4A001 Electronic computers and related equipment, having any of the following (see List of Items Controlled), and “electronic assemblies” and “specially designed” “components” therefor.

License Requirements

Reason for Control:	NS, MT, AT, NP

| Control(s) | Country Chart |  |
| --- | --- | --- |
|  | (See Supp. No. |  |

|  |  | 1 to part 738) |
| --- | --- | --- |
| NS applies to entire entry |  | NS Column 2 |
| MT applies to items in |  | MT Column 1 |
| 4A001.a when the parameters |  |  |
| in 4A101 are met or exceeded |  |  |
| AT applies to entire entry |  | AT Column 1 |
| NP applies, unless a License |  | N/A |
| Exception is | available. See |  |
| §742.3(b) of the EAR for |  |  |
| information | on applicable |  |
| licensing review policies. |  |  |

Reporting Requirements

See § 743.1 of the EAR for reporting requirements for exports under License Exceptions, and Validated End-User authorizations.

List Based License Exceptions (See Part 740 for a description of all license exceptions)

LVS:	$5000 for 4A001.a; N/A for MT.

GBS:	N/A

CIV:	N/A

Special Conditions for STA

STA:	License Exception STA may not be used to ship any commodity in 4A001.a.2 to any of the destinations listed in Country Group A:6 (See Supplement No.1 to part 740 of the EAR).

List of Items Controlled

Related Controls: See also 4A101 and 4A994. Equipment designed or rated for transient ionizing radiation is “subject to the ITAR” (see 22 CFR parts 120 through 130).

Related Definitions: For the purposes of integrated circuits in 4A001.a.2, 5 x 103 Gy(Si) = 5 x 105 Rads (Si); 5 x 106 Gy (Si)/s = 5 x 108 Rads (Si)/s.

Items:

Export Administration Regulations	Bureau of Industry and Security	October 24, 2018

Commerce Control List	Supplement No. 1 to Part 774	Category 4—page 2

a. “Specially designed” to have any of the following:

a.1. Rated for operation at an ambient temperature below 228 K (-45˚C) or above 358 K (85˚C); or

Note: 4A001.a.1 does not apply to computers “specially designed” for civil automobile, railway train or “civil aircraft” applications.

a.2. Radiation hardened to exceed any of the following specifications:

a.2.a. A total dose of 5 x 103 Gy (Si);

a.2.b. A dose rate upset of 5 x 106 Gy (Si)/s; or

a.2.c. Single Event Upset of 1 x 10-8 Error/bit/day;

Note: 4A001.a.2 does not apply to computers “specially designed” for “civil aircraft” applications.

b. [Reserved]

4A003 “Digital computers”, “electronic assemblies”, and related equipment therefor, as follows (see List of Items Controlled) and “specially designed” “components” therefor.

License Requirements

Reason for Control: NS, CC, AT

|  | Country Chart (See |
| --- | --- |
| Control(s) | Supp. No. 1 to part |
|  | 738) |
| NS applies to 4A003.b and .c | NS Column 1 |
| NS applies to 4A003.e and .g | NS Column 2 |
| CC applies to “digital | CC Column 1 |
| computers” for computerized |  |

finger-print equipment

AT applies to entire entry AT Column 1 (refer to 4A994 for controls

on “digital computers” with a APP > 0.0128 but ≤ 29 WT)

Note: : For all destinations, except those countries in Country Group E:1 or E:2 of Supplement No. 1 to part 740 of the EAR, no license is required (NLR) for computers with an “Adjusted Peak Performance” (“APP”) not exceeding 29 Weighted TeraFLOPS (WT) and for “electronic assemblies” described in 4A003.c that are not capable of exceeding an “Adjusted Peak Performance” (“APP”) exceeding 29 Weighted TeraFLOPS (WT) in aggregation, except certain transfers as set forth in § 746.3 (Iraq).

Reporting Requirements

Special Post Shipment Verification reporting and recordkeeping requirements for exports of computers to destinations in Computer Tier 3 may be found in §743.2 of the EAR.

List Based License Exceptions (See Part 740 for a description of all license exceptions)

LVS:	$5000; N/A for 4A003.b and .c.

GBS:	Yes,  for  4A003.g  and  “specially

designed” “parts” and “components” therefor, exported separately or as part of a system.

APP:	Yes,  for  computers  controlled  by

4A003.b, and “electronic assemblies” controlled by 4A003.c, to the exclusion of other technical parameters. See §740.7 of the EAR.

CIV:	Yes, for 4A003.g.

List of Items Controlled

Related Controls: See also 4A994 and 4A980

Related Definitions: N/A

Items:

Export Administration Regulations	Bureau of Industry and Security	October 24, 2018

Commerce Control List	Supplement No. 1 to Part 774	Category 4—page 3

Note 1: 4A003 includes the following:

- ‘Vector processors’ (as defined in Note 7 of the “Technical Note on “Adjusted Peak Performance” (“APP”)”);

- Array processors;

- Digital signal processors;

- Logic processors;

- Equipment designed for “image enhancement.”

Note 2: The control status of the “digital computers” and related equipment described in 4A003 is determined by the control status of other equipment or systems provided:

a. The “digital computers” or related equipment are essential for the operation of the other equipment or systems;

b. The “digital computers” or related equipment are not a “principal element” of the other equipment or systems; and

N.B. 1: The control status of “signal processing” or “image enhancement” equipment “specially designed” for other equipment with functions limited to those required for the other equipment is determined by the control status of the other equipment even if it exceeds the “principal element” criterion.

N.B. 2: For the control status of “digital computers” or related equipment for telecommunications equipment, see Category 5, Part 1 (Telecommunications).

  - The “technology” for the “digital computers” and related equipment is determined by 4E.

- [Reserved]

- “Digital computers” having an “Adjusted Peak Performance” (“APP”) exceeding 29 Weighted TeraFLOPS (WT);

- “Electronic assemblies” “specially designed” or modified to be capable of enhancing performance by aggregation of processors so that the “APP” of the aggregation exceeds the limit in 4A003.b.;

Note 1: 4A003.c applies only to “electronic

assemblies” and programmable interconnections not exceeding the limit in 4A003.b when shipped as unintegrated “electronic assemblies.”

Note 2: 4A003.c does not control “electronic assemblies” “specially designed” for a product or family of products whose maximum configuration does not exceed the limit of 4A003.b.

d. to f.  [Reserved]

N.B.: For “electronic assemblies,” modules or equipment, performing analog-to-digital conversions, see 3A002.h.

- Equipment “specially designed” for aggregating the performance of “digital

computers” by providing external interconnections which allow communications at unidirectonal data rates exceeding 2.0 Gbyte/s per link.

Note: 4A003.g does not control internal interconnection equipment (e.g., backplanes, buses) passive interconnection equipment,

“network access controllers” or “communication channel controllers”.

4A004 Computers as follows (see List of Items Controlled) and “specially designed” related equipment, “electronic assemblies” and “components” therefor.

Export Administration Regulations	Bureau of Industry and Security	October 24, 2018

Commerce Control List	Supplement No. 1 to Part 774	Category 4—page 4

License Requirements

Reason for Control:	NS, AT

|  | Country Chart |
| --- | --- |
| Control(s) | (See Supp. No. 1 |
|  | to part 738) |
| NS applies to entire entry | NS Column 2 |
| AT applies to entire entry | AT Column 1 |

List Based License Exceptions (See Part 740 for a description of all license exceptions)

LVS:	$5000

GBS:	N/A

CIV:	N/A

List of Items Controlled

Related Controls: N/A

Related Definitions: N/A

Items:

a. ‘Systolic array computers’; b. ‘Neural computers’; c. ‘Optical computers’. Technical Notes:

1. ‘Systolic array computers’ are computers where the flow and modification of the data is dynamically controllable at the logic gate level by the user.

- ‘Neural computers’ are computational devices designed or modified to mimic the behaviour of a neuron or a collection of neurons, i.e., computational devices which are distinguished by their hardware capability to modulate the weights and numbers of the interconnections of a multiplicity of computational components based on previous data.

- ‘Optical computers’ are computers designed or modified to use light to represent data and whose computational logic elements are based on directly coupled optical devices.

4A101 Analog computers, “digital computers” or digital differential analyzers, other than those controlled by 4A001 designed or modified for use in “missiles”, having any of the following (see List of Items Controlled).

License Requirements

Reason for Control:	MT, AT

|  | Country Chart |
| --- | --- |
| Control(s) | (See Supp. No. 1 |
|  | to part 738) |
| MT applies to entire entry | MT Column 1 |
| AT applies to entire entry | AT Column 1 |

List Based License Exceptions (See Part 740 for a description of all license exceptions)

LVS:	N/A

GBS:	N/A

CIV:	N/A

List of Items Controlled

Related Controls: N/A

Related Definitions: N/A

Items:

- Rated for continuous operation at temperatures from below 228 K (-45o C) to above 328 K (+55oC); or

- Designed as ruggedized or ‘radiation hardened’.

Note: ‘Radiation hardened’ means that the “part,” “component” or equipment is designed or rated to withstand radiation levels which meet or exceed a total irradiation dose of 5 X

Export Administration Regulations	Bureau of Industry and Security	October 24, 2018

Commerce Control List	Supplement No. 1 to Part 774	Category 4—page 5

105 rads (Si).

4A102 “Hybrid computers” “specially designed” for modelling, simulation or design integration of “missiles” or their subsystems. (These items are “subject to the ITAR” See 22 CFR parts 120 through 130.)

4A611  Computers,	and	“parts,”

“components,”	“accessories,”	and

“attachments” “specially designed” therefor, “specially designed” for a military application that are not enumerated in any USML category are controlled by ECCN 3A611.

4A980 Computers for fingerprint equipment, n.e.s.

License Requirements

Reason for Control:	CC, AT

|  | Country Chart |
| --- | --- |
| Control(s) | (See Supp. No. 1 |
|  | to part 738) |
| CC applies to entire entry | CC Column 1 |
| AT applies to entire entry | AT Column 1 |

List Based License Exceptions (See Part 740 for a description of all license exceptions)

LVS:	N/A

GBS:	N/A

CIV:	N/A

List of Items Controlled

Related Controls: N/A

Related Definitions: N/A

Items:

The list of items controlled is contained in the ECCN heading.

Note: 4A980 does not control equipment limited to one finger and designed for user authentication or access control.

4A994 Computers, “electronic assemblies” and related equipment, not controlled by 4A001 or 4A003, and “specially designed” “parts” and “components” therefor (see List of Items Controlled).

License Requirements

Reason for Control:	AT

|  | Country Chart |
| --- | --- |
| Control(s) | (See Supp. No. 1 |
|  | to part 738) |
| AT applies to entire entry | AT Column 1 |

List Based License Exceptions (See Part 740 for a description of all license exceptions)

LVS:	N/A

GBS:	N/A

CIV:	N/A

List of Items Controlled

Related Controls: N/A

Related Definitions: N/A

Items:

Note 1: The control status of the “digital computers” and related equipment described in 4A994 is determined by the control status of other equipment or systems provided:

a. The “digital computers” or related equipment are essential for the operation of the other equipment or systems;

b. The “digital computers” or related equipment are not a “principal element” of the other equipment or systems; and

Export Administration Regulations	Bureau of Industry and Security	October 24, 2018

Commerce Control List	Supplement No. 1 to Part 774	Category 4—page 6

N.B. 1: The control status of “signal processing” or “image enhancement” equipment “specially designed” for other equipment with functions limited to those required for the other equipment is determined by the control status of the other equipment even if it exceeds the “principal element” criterion.

N.B. 2: For the control status of “digital computers” or related equipment for telecommunications equipment, see Category 5, Part 1 (Telecommunications).

c. The “technology” for the “digital computers” and related equipment is determined by 4E.

a. Electronic computers and related equipment, and “electronic assemblies” and “specially designed” “parts” and “components” therefor, rated for operation at an ambient temperature above 343 K (70°C);

b. “Digital computers”, including equipment of “signal processing” or image enhancement”, having an “Adjusted Peak Performance” (“APP”) equal to or greater than 0.0128 Weighted TeraFLOPS (WT);

c. “Electronic assemblies” that are “specially designed” or modified to enhance performance by aggregation of processors, as follows:

c.1. Designed to be capable of aggregation in configurations of 16 or more processors;

c.2. [Reserved];

Note 1: 4A994.c applies only to “electronic

assemblies” and programmable interconnections with a “APP” not exceeding the limits in 4A994.b, when shipped as unintegrated “electronic assemblies”. It does not apply to “electronic assemblies” inherently limited by nature of their design for use as related equipment controlled by 4A994.k.

Note 2: 4A994.c does not control any “electronic assembly” “specially designed” for a product or family of products whose maximum configuration does not exceed the limits of 4A994.b.

- [Reserved];

- [Reserved];

- Equipment for “signal processing” or “image enhancement” having an “Adjusted Peak Performance” (“APP”) equal to or greater than 0.0128 Weighted TeraFLOPS WT;

- [Reserved];

- [Reserved];

- Equipment containing “terminal interface equipment” exceeding the limits in 5A991;

- Equipment “specially designed” to provide external interconnection of “digital computers” or associated equipment that allows communications at data rates exceeding 80 Mbyte/s.

Note: 4A994.j does not control internal interconnection equipment (e.g., backplanes, buses) passive interconnection equipment,

“network access controllers” or “communication channel controllers”.

k. “Hybrid computers” and “electronic assemblies” and “specially designed” “parts” and “components” therefor containing analog-to-digital converters having all of the following characteristics:

k.1.  32 channels or more; and

k.2. A resolution of 14 bit (plus sign bit) or more with a conversion rate of 200,000 conversions/s or more.

Export Administration Regulations	Bureau of Industry and Security	October 24, 2018